OBJECTIVES OF LABS

- Part 1: Build the Network and Verify Connectivity

- Part 2: Use a NETCONF Session to Gather Information

- Part 3: Use ncclient to Connect to NETCONF

- Part 4: Use ncclient to Retrieve Running Configuration (get-config)

- Part 5: Use ncclient to Edit Configuration (edit-config)

# Netconf

Netconf is a network monitoring protocol which configure network devices like routers, switches programmatically. It removes configuration complexity, vendor specific commands and have better efficiency which is generally found in SNMP.

- Data representation: XML

- Protocol used: SSH

- Alternative: RestConf

- Features:

  - Device configuration management (CRUD of devices)

  - Transactional changes (support commit and rollback)

  - Standardized and structured with Yang model

- Benefits:

  - Automation-Ready (SDN and automation tool like ansible etc)

  - Standardized (Yang models)

  - Error Handling (validation and rollback)

  - Security (due to SSH)

- Disadvantages:

  - Complexity (XML and other learning curve)

  - Performance overhead (XML And SSH compare to json and Rest features)

  - Scalability (separate SSH connection for each SSH connection, which limits how many simultaneous sessions a server can manage.)

- Useful Commands:

  - get - Retrieve operational data e.g., interface status.

  - get-config - Retrieve configuration data.

  - edit-config - Modify the device configuration.

  - commit - Apply candidate configuration to the running state.

  - discard-changes - Discard uncommitted changes.

  - lock and unlock - Prevent concurrent edits during a session.

  - delete-config - Delete a specified data store (e.g., candidate).

  - close-session

- What it solves:

  - Automated Configuration

  - Transaction Integrity

  - Scalability

  - Efficiency

  - Example: In a large network, manually updating IP addresses on hundreds of routers is inefficient. NETCONF allows a centralized script to update all routers automatically using programs.

- Tools Required:

  - GNS3

  - Router with capability to integrate with python (e.g., CSR 1000v)

  - Python with ncclient

  - GitHub

# Part 1: Build the Network and Verify Connectivity:

- Topology:

  - Cisco csr 1000v connected to cloud with Giga Ethernet.

  - Cloud side is configured with br0.

- Setting up clock

- Interfaces:

  - DHCP automatically assign Ip address to connected interface(gi1)

- Configure SSH with cisco csr 1000v:

  - Basic Configuration for password and banners

  - Creating a username and provide privileges to user to all tasks.

  - Now generate a RSA (Rivest-Shamir-Adleman) key pairs, which is essential for enabling secure communication protocols such as SSH (Secure Shell) and HTTPS.

  - Configure “ssh version 2” and then enter into virtual terminal.

  - VTY lines control remote access (Telnet/SSH) to the Cisco device.

  - “Transport input ssh” Restricts the transport protocol to SSH only.

  - login local defines that Configures the router to authenticate remote users using local credentials

  - Verify User can access router remotely with SSH credentials.

# Part 2: Use a NETCONF Session to Gather Information

  - Enable Netconf in router:

  - Verify router is enabled for Netconf

  - Verify Netconf connection

# Part 3: Use ncclient to Connect to NETCONF

- Using manager from ncclient library  to connect with Router and verify with server capability.

# Part 4: Use ncclient to Retrieve Running Configuration (get-config)

  - Python function for get_config

  - Generated xml file for get config

# Part 5: Use ncclient to Edit Configuration (edit-config)

  - Python script for edit config

  - It creates a loopback address with user input for name ip and subnet mask

  - User response on excuting the add_a_loopback(m) function:

  - Verify in Gns3